www.chipsee.com

User Manual

Release 1.0

CS10768F097E-C111

CS10768F097P-C111

Table of Contents

Table of Contents	2

Chipsee Products Naming Rules	3

Hardware Features	4

CS10768F097E-C111	5

CS10768F097P-C111	6

Power Input Connector	7

Capacitive Touch	7

CAN+RS485+RS232 Connector	8

USB HOST Connector	9

USB OTG Connector	9

LAN Connector	9

TF Card	10

SIM Card Holder	10

Audio Connector	11

WiFi+BT	11

HDMI Connector	11

Boot Switch	12

GPIO Connector	13

Measurements and Mounting	14

Measurements of CS10768F097E	14

Mounting Method of CS10768F097E	14

Measurements of CS10768F097P	15

Mounting Method of CS10768F097P	15

How to Get Support	16

Chipsee Products Naming Rules

| CS10768F097E-C111 CS10768F097P-C111 |  |
| --- | --- |
| CS | Chipsee Product Abbreviate |
| 10 | Horizontal Resolution 80 Means 800 Pixel 10 Means 1024 Pixel 12 Means 1280 Pixel 14 Means 1440 Pixel 19 Means 1920 Pixel |
| 768 | Vertical Resolution 480 Means 480 Pixel 600 Means 600 Pixel 768 Means 768 Pixel 800 Means 800 Pixel 900 Means 900 Pixel 102 Means 1024 Pixel 108 Means 1080 Pixel |
| F(T) | Product based on Freescale (TI) CPU |
| 097 | LCD Dimension 050 Means 5.0 Inch 070 Means 7.0 Inch 080 Means 8.0 Inch 097 Means 9.7 Inch 101 Means 10.1 Inch 104 Means 10.4 Inch 120 Means 12.0 Inch 150 Means 15.0 Inch 170 Means 17.0 Inch 190 Means 19.0 Inch 215 Means 21.5 Inch |
| E | Means Embedded PC or Panel PC E Means Embedded PC without Case P Means Panel PC with Case |
| C | Means Touch Type R Means Resistive Touch C Means Capacitive Touch |
| 1 | Means LCD Brightness 1 Means Common Brightness 2 Means High Brightness |
| 1 | PCB Version Baseboard PCB Version Number |
| 1 | PCB Version SOM Module PCB Version Number |

Hardware Features

| Key Features: |  |
| --- | --- |
| CPU | iMX6Q,Quad-A9,1GHz |
| RAM | 2GB DDR3 |
| eMMC | 8GB |
| Storage | TF card, supports up to 32GB SDHC |
| Display | 9.7 Inch LCD,1024*768 Pixel, Brightness:350nit |
| Touch | Ten-Point Capacitive Touch |
| USB | 2 x USB 2.0 Host,1 USB OTG |
| LAN | 1 Channel 1000M LAN, Support POE（Optional）. 2nd Channel 100M Optional |
| Audio | 3.5mm Audio In/Out Connector,2W Speaker Internal |
| Buzzer | 1 |
| RTC | Yes |
| RS232 | 2 Channels |
| RS485 | 3 Channels * |
| CAN | 2 Channels |
| GPIO | 8 Channels |
| WiFi/BT | On Board WIFI/BT |
| HDMI | 1 Channel |
| SATA II | 1 Channel |
| 3G/4G | Optional, Not Mount on by default. |
| Power Input | 12~36V DC |
| Current @ 15V | 600 mA max(No 4G module) |
| Power Consumption | 7W Typical |
| Working Temperature | -20°C to +70°C |
| OS | Android, Ubuntu, Linux |
| Dimension | CS10768F097E: 226*172*26mm CS10768F097P:256*207*29mm |
| Weight | CS10768F097E:550g CS10768F097P:1370g |

* This product has 5 channels of UART in total. The Bluetooth used one channel UART by default, the default setting is 2*RS232+2*RS485+Bluetooth.The UART can be changed between RS232 and RS485 easily. If you want different RS232 and RS485 setting, please contact us.

CS10768F097E-C111

Figure 1 Top View (Ubuntu)

Figure 2 Back View

CS10768F097P-C111

Figure 3 Top View (Android)

Figure 4 Back View

Power Input Connector

The product CS10768F097E/CS10768F097P uses a wide range power input: DC 12~36V. The total power consumption is about 7W normally. The Power Input Connector is 3 Pin 3.81mm Screw Terminal Connector as Figure 5 shows. The Character “+” means power Positive input, The Character “-” means power Negative input. The Character “G” means system Ground. Table 1 has detailed descriptions about the connector definition.

Figure 5 Power Input Connector

Table 1

| Power Input Pin Definition: |  |  |
| --- | --- | --- |
| Pin Number | Definition | Description |
| Pin 1 | Positive Input | Connect to DC Power Positive Terminal |
| Pin 2 | Negative Input | Connect to DC Power Negative Terminal |
| Pin 3 | Ground | Connect to Power System Ground |

BE ATTENTION:

The system ground “G” has been connected to power negative “-” on board.

Capacitive Touch

The product CS10768F097E/CS10768F097P uses ten-point capacitive touch.

BE ATTENTION:

Capacitive touch is very sensitive to power noise. Ripple voltage/current from the power adapter will cause the LCD ripples, and will also cause the capacitive touch malfunction: If you use the APK Multi-Touch under Android to test, you can find the touch point float. There are several ways to solve this problem:

1)  Use a high quality power adapter. Or use battery to provide the power like cell phone or tablet PC.

2)  If user power adapter can’t be good enough, there’s another effective method to solve this problem: Make sure the power input connector Pin 3 really connect to user “Power System Ground”. This method can eliminate the problem totally. User can also use another method to test this problem: touch the GND of CS10768F097P-C111 by one hand, the other hand operates on the capacitive touch screen. In this case, user’s body acts as the Power System Ground.

CAN+RS485+RS232 Connector

The CAN+RS485+RS232 connector is a 16 Pin 3.81mm Screw Terminal Connector, as Figure 6 shows. As for the definition of every pin, please refer to Table 2.This product has 5 channels of UART in total, they can be configured as RS232 or RS485 freely. If you need any other setting different to the default setting, please contact us.

Figure 6 CAN+RS485+RS232 Connector

Table 2

| RS232 / RS485 / CAN Pin Definition: |  |  |
| --- | --- | --- |
| Pin Number | Definition | Description |
| Pin 1 | CAN2_H | CPU CAN Channel 2 H signal |
| Pin 2 | CAN2_L | CPU CAN Channel 2 L signal |
| Pin 3 | CAN1_H | CPU CAN Channel 1 H signal |
| Pin 4 | CAN1_L | CPU CAN Channel 1 L signal |
| Pin 5 | RS485_2- | CPU UART2, RS485 -(A) signal |
| Pin 6 | RS485_2+ | CPU UART2, RS485 +(B) signal |
| Pin 7 | RS485_5- | CPU UART5, RS485 -(A) signal |
| Pin 8 | RS485_5+ | CPU UART5, RS485 +(B) signal |
| Pin 9 | RS485_4- | CPU UART4, RS485 -(A) signal |
| Pin 10 | RS485_4+ | CPU UART4, RS485 +(B) signal |
| Pin 11 | RS232_3_RXD | CPU UART3, RS232 RXD signal |
| Pin 12 | RS232_3_TXD | CPU UART3, RS232 TXD signal |
| Pin 13 | RS232_1_RXD | CPU UART1, RS232 RXD signal |
| Pin 14 | RS232_1_TXD | CPU UART1, RS232 TXD signal |
| Pin 15 | GND | System Ground |
| Pin 16 | +5V | System +5V Power Output, No more than 1A Current output. |

BE ATTENTION:

1. UART2 signal has been used by Bluetooth signal on board. The RS485_2 function has been disabled by default. If customer needs to use RS485_2 function, please contact us, we will disable the Bluetooth function.

2. All the CAN signal and RS485 signal have not mount on the 120Ohm Matched Resistance.

USB HOST Connector

The product CS10768F097E/CS10768F097P has two USB connectors as Figure 7 shows. These two USB can provide 500mA current each.

Figure 7 USB HOST Connector

USB OTG Connector

The product CS10768F097E/CS10768F097P has one USB OTG connector as Figure 8 shows. It works as slave by default. User can connect it to host PC by this connector.

Figure 8 USB OTG Connector

LAN Connector

The product CS10768F097E/CS10768F097P has one channel 1000Mbit Ethernet Connector(LAN1), as Figure 9 shows. And this port support POE(Power-On-Ethernet, optional, not supported by default).The other one channel 100Mbit Ethernet Connector is optional.

Figure 9 LAN Connector

TF Card

The product CS10768F097E/CS10768F097P has one TF(uSD) card connector as Figure 10 shows. This device supports TF(uSD) card up to 32GB.

Figure 10 TF Card Connector

BE ATTENTION:

The TF card slot is NOT mounted with any TF card by default.

SIM Card Holder

The product CS10768F097E/CS10768F097P has a mini-PCIe connector inside, customer can mount on 4G module to it. Then it will need the SIM Card Holder, as Figure 12 shows. There is a connector on the backside case which can connect external 4G Antenna, as Figure 13 shows.

Figure 11mini-PCIe Connector&4G Module

Figure 12 SIM Card Holder

Figure 13 4G Antenna

BE ATTENTION:

This product is not mounted on any 4G module by default. And Ubuntu driver for 4G module is not available at present.

Audio Connector

The product CS10768F097E/CS10768F097P has one Audio Input (“Line-in”) and one Audio (“Line-out”) output, as Figure 14 shows. And the product has an internal 2W speaker.

Figure 14 Audio Connector

WiFi+BT

The product CS10768F097E/CS10768F097P has one WiFi+BT. It uses Realtech RTL8723 which integrates WiFi and BT. There is a connector on the backside case which can connect external WiFi/BT Antenna, as Figure 16 shows.

Figure 15 WiFi+BT Module

Figure 16 WiFi+BT Antenna

HDMI Connector

The product CS10768F097E/CS10768F097P has one HDMI connector as Figure 17 shows. The HDMI output resolution can be configured by software.

Figure 17 HDMI Connector

Boot Switch

The product CS10768F097E/CS10768F097P has a boot switch which can be used to change boot sequence, as Figure 18 shows. It is defined as SW2 on the PCB. As for the details of the boot switch, please refer to Table 3.

Figure 18 Boot Switch

Table 3

| Boot Config Select: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Mode | 1 | 2 | 3 | 4 |
| TF Card | 1 | 0 | 0 | 0 |
| eMMC | 1 | 1 | 0 | 1 |
| Download | 0 | 1 | 1 | 0 |

GPIO Connector

The product CS10768F097P-R141 has one GPIO Connector, as Figure 19 shows. This connector is labelled as P28 on the PCB.. As for the definition of every Pin, please refer to Table 4.

Figure 19 GPIO Connector

Table 4

| GPIO Connector Definition |  |
| --- | --- |
| Pin Number | Definition |
| Pin 1 | VCC, Isolated Power 5VInput |
| Pin 2 | GND |
| Pin 3 | OUT1 |
| Pin 4 | OUT2 |
| Pin 5 | OUT3 |
| Pin 6 | OUT4 |
| Pin 7 | IN1 |
| Pin 8 | IN2 |
| Pin 9 | IN3 |
| Pin 10 | IN4 |

Measurements and Mounting

Measurements of CS10768F097E

The measurement of CS10768F097E-C111 is 226*172*26mm.

Mounting Method of CS10768F097E

This product CS10768F097E-C111 can be mounted using the 4 screw holes on the PCB. Please make sure the display is not exposed to high pressure when mounting into an enclosure.

Measurements of CS10768F097P

The measurement of CS10768F097P-C111 is 256*207*29mm.

Mounting Method of CS10768F097P

This product CS10768F097P-C111 can be mounted using the mounting set, as Figure 20 shows. Please make sure the display is not exposed to high pressure when mounting into an enclosure.

Figure 20 Mounting Method

How to Get Support

Please feel free to contact us with any questions, queries or suggestions.

If your question is about technical support or troubleshooting for one of our products, we kindly ask you to first check our documentation for a possible solution.

If you cannot find the solution you are looking for then please write to service@chipsee.com providing all possible details.

© Chipsee all right reserved.

Chipsee assumes no responsibility for any errors, which may appear in this manual. Furthermore, Chipsee reserves the right to alter the hardware, software, and/or specifications detailed herein at any time without notice, and does not make any commitment to update the information contained herein. Chipsee products are not authorized for use as critical components in life support devices or systems.